MPhil / PhD Projec

Ref:

Please save this document using your surname (e.g. ResProj-surname.doc ) and return to pgeng@liv.ac.uk as an attachment. Thank you.  Dr H Ouyang, Postgraduate Admissions Tutor.

| Supervisor(s) | Dr A T Shenton | ### Suitable for (MPhil, MPhil/PhD, PhD) | PhD |
| --- | --- | --- | --- |
| Project Title | IC Engine Characterisation, Modelling, Simulation & Control With Add2 Ltd. |  |  |
| ## Project Description |  |  |  |
| Modern methods for developing controls for automotive engines are based on replacing parts of the engine hardware with computer controlled emulators. These are termed ‘hardware in-the-loop (HIL) systems. They allow automotive companies to speed up the calibration (ie ‘tuning’) & testing of engine computer controls. The process is called rapid control prototyping (RCP) Add2 Ltd is a leading company supplying such equipment to major automotive manufacturers such as Jaguar & Ford. Add2 Ltd will supply hardware & software to support testing, electronics support & sharing of information on the dSPIC microcontroller, & engineering time & support. The objective of this project would be to characterise the IC engine in a generic way to create a standard list of signals (Inputs to & outputs from the engine) & to explore how these signals can be emulated/reproduced for HIL & RCP development activities during the Control System development cycle. The work would involve a number of stages: 1 Characterisation - Identify engine configurations (i.e. variants such as PFI, GDi, TIVCT, HCCI?) - Develop a ‘standard’ list of I/O signals for engine sensors & actuators. - Where possible, capture representative signals for I/O at different operating ranges for 1 & 4 cylinder engines. 2 Modelling - Waveform generation – ‘arbitary’ waveform generation for engine simulation. Utilise this technology for generating System ID inputs. - System Identification utilising data captured in characterisation activities. - Physical models (simple) validated against data & system ID models. Explore use of Simulink vs OpenModelica. 3 Controllers - Controller design – use models (from system ID & physical modelling) to design controllers. - Implement controllers – use RCP (add2 & dSPACE) - Evaluate performance on dynamometer - Evaluate performance on HIL 4 Outputs/Dissemination - An ‘index’ of engine I/O signals with characteristics will be produced Applicants should have an interest in developing their computer hardware & software skills & should have an interest in control theory. The project will involve extensive use of Matlab/Simulink Real-Time-Workshop & dSPACE control systems. The skill sets that will be acquired during the project are currently in shortage & in great demand in industry. The project will use the existing Liverpool Engine-Dynamometer & Chassis-Dynamometer facilities in the Liverpool Powertrain Control Laboratory. For more details about the Powertrain Control Laboratory see: http://www.liv.ac.uk/engdept/powertraincontrol/ |  |  |  |

| Funding |  |  |  |  |
| --- | --- | --- | --- | --- |
| ### Source |  |  | Secured? (yes/no) |  |
| Funding details |  |  |  |  |

| Request for support by Departmental EPSRC funds? (yes/ no) |  |
| --- | --- |

Candidate: special qualities/background required (e.g. 1st degree must include fluids)

| Submitted by |  | Date |  |
| --- | --- | --- | --- |