|  | # Radiation Protection Service DUTIES OF PERSONNEL WORKING WITH IONISING RADIATION |
| --- | --- |

Note: The ultimate responsibility for radiation safety and compliance with radiation legislation rests with the Vice-Chancellor. This responsibility can not be delegated to the RPS or to the RPA.

# Role of the University’s Radiation Protection Advisor

Overall supervision and advice on radiological safety is the responsibility of the University’s Radiation Protection Advisor (RPA). The RPAs are Pete Cole and Louise Nicholson – phone 0151 794 3467 e-mail pcole@liv.ac.uk and louise.nicholson@liv.ac.uk

The RPA must be consulted on certain issues but the ultimate responsibility for radiation safety in the University rests with the Vice-Chancellor. At a departmental level, responsibility for radiation safety rests with the Head of Department.  In addition, individuals within University departments who are working with radiation have a duty to work safely and legally.

The RPA must be consulted with in relation to the following matters;

- The examination and testing of protective devices and measuring instruments

- The prior critical examination of plans for installations from the point of view of radiation protection

- The acceptance into service of new or modified sources from the point of view of radiation protection

- The regular checking of the effectiveness of protective devices and techniques

- The regular calibration of measuring instruments, and the regular checking that they are serviceable and correctly used

The duties of the RPA include the following;

- To provide information and advice on all aspects of radiation protection in The University of Liverpool main campus and its ‘satellite’ sites i.e. Leahurst Campus and University labs at RLBUHT

- To aim to ensure that radiation exposures are kept As Low As Reasonably Practicable (ALARP) by co-ordinating the radiation protection service in the University.

- To ensure that accurate records are maintained of personnel exposure and of incidents involving exposure to radiation, radioactive contamination or of any radiological significance.

- To ensure that appropriate instruction is given in the proper procedures to be used and in the use of equipment necessary for safe working with ionising radiation.

- To ensure that monitoring equipment is regularly checked and calibrated.

- To ensure that records are maintained of receipt, usage, storage and disposal of radioactive materials as may be required by the Environment Agency.

- To ensure that all users of radioactive materials are informed of appropriate safety rules and national legislation as may be applicable.

- To carry out such other duties concerning radiation safety as may be required by the Consultative Committee on Health and Safety.

The RPA is authorised to refuse permission to undertake work with ionising radiation in any University department if s/he is not satisfied that the necessary safety and legal compliance requirements can be met. The RPA is also authorised to require that any work activity with ionising radiation, which s/he deems to be unsafe, or in contravention of the University’s Permit requirements, or in contravention of the University’s Radiation Safety Code of Practice, cease or be suspended until appropriate control measures are implemented.

# Role of the Departmental Radiation Protection Supervisor (RPS)

The primary role of the RPS is to ensure, at a departmental level, day-to-day compliance with the requirements of the Ionising Radiations Regulations 2017, other relevant radiation legislation, the University’s Permits provisions, the University’s Radiation Safety Code of Practice and Departmental Local Radiation Safety Rules, by personnel working with radiation within their department.

This includes ensuring that the following general arrangements are implemented;

- Ensuring that all radioactive sources are labelled, segregated from other substances, and kept in a safe and secure location

- Ensuring that a warning system is in place for rooms containing irradiating apparatus, that there is appropriate signposting of radiochemical labs with trefoil signs, and signposting and control of controlled and supervised areas

- Ensuring that a copy of the Departmental Local Radiation Safety Rules are displayed within the department and ensuring that all users are aware of these and other relevant information, which is passed to the RPS by the RPA or Consultative Committee on H&S.

- Ensuring that sources of ionising radiation or irradiating apparatus are only acquired in the department with the prior approval of the RPS and RPA

- Ensuring where appropriate that dosimetry badges are provided to all personnel likely to be exposed to ionising radiation, that adequate records are maintained, and that incidents likely to lead to significant levels of exposure are investigated, and reported to the RPA

- Ensuring that routine checks and maintenance are carried out on irradiating equipment, in accordance with manufacturer's instructions, and ensuring that no modifications are undertaken to irradiating equipment without the prior approval of the RPA

- Ensuring that leak tests of sealed sources are undertaken every 2 years

- Ensuring that adequate contamination monitoring programmes are implemented in unsealed radioactive material areas and ensuring that contamination levels are kept ALARP

- Ensuring that no radioactive source or irradiating apparatus is disposed of without the prior approval of the RPS and the RPA/Radioactive Waste Adviser (RWA), and ensuring that disposal is only undertaken in accordance with agreed and approved procedures.

- Arranging for the disposal of all departmental radioactive waste in accordance with approved procedures and liaising with the RPA/RWA as necessary. Keeping adequate records of all radioactive waste disposed of directly from the department

- Keeping all relevant records as required by the University’s EPR Permits and any other relevant radiation legislation, and having such records available for inspection by the RPA or regulatory bodies

- Exercising control over the acquisition and disposal of sources, maintaining an inventory of sources within the department, and ensuring that there are no unrecorded sources in the department. Also to liaise with the University’s RPA/RWA on quantities of radioactive materials brought into the department (using form RP2A) and specifically if this is to exceed by more than 50% the department’s baseline figure for radioactive material holdings.

- Keeping records of the quantity and nature of each radioisotope plus the usage and disposal of each source. Providing a monthly statement to the RPO on what radioactive sources were acquired in the previous month, what radioactive waste was disposed of and how much activity is held in stock at the end of each month.

- Ensuring that all new projects involving the use of radiation within the department receive the prior approval of the RPA

- Ensuring that all departmental personnel working with radiation are registered as radiation workers using the RPO’s online database

- Ensuring that all departmental personnel working with radiation are aware of the University’s EPR Permit conditions and schedules plus the University’s Radiation Safety Code of Practice, and comply with competence requirements

- Ensuring that the department has an adequate number of monitoring instruments and that they are calibrated regularly (every 12 months)

- Ensuring that persons working with unsealed sources wear appropriate PPE e.g. gloves, safety glasses, and lab coats and that persons working with unsealed sources in volatile/powder form only work in a fume cupboard

- Taking appropriate measures on being notified of spillages, and reporting such spillage events to the RPA/RWA forthwith

# Role of the Individual Radiation Worker

Each individual in the University working with ionising radiation has a responsibility for taking all due care for their own health and safety and the health and safety of anyone who may be affected by their work activities.

All radiation workers in the University are obliged to familiarise themselves with and to comply with the conditions of the University’s Local Radiation Safety Rules for the Work with Ionising Radiations within their Department.

All radiation workers in University are obliged to consult with their RPS before undertaking any work with ionising radiation, and as necessary during the course of their work.

All radiation workers in University are obliged to co-operate with their RPS in complying with the provisions of the Local Rules, and are obliged to comply with any recommendations or advice given by the RPS.

In particular all radiation workers are obliged to:

- Ensure that all radioactive sources are labelled, segregated from other substances, and kept in a safe and secure location.

- Register as a radiation worker using the RPO’s online database

- Ensure that sources of ionising radiation or irradiating apparatus are only acquired in the department with the prior approval of the RPS.

- Ensure that dosimetry badges, if issued, are worn and returned promptly at the end of each issue period

- Ensure that no modifications are undertaken to irradiating equipment without the prior approval of the RPS and RPA

- Ensure that leak tests of sealed sources within their control are undertaken every 2 years.

- Ensure that an adequate contamination monitoring procedures are undertaken in work areas where unsealed radioactive materials are handled and ensuring that contamination levels are kept ALARP

- Keep records of the quantity and nature of each radioisotope under their control

- Keep records of the usage of each source under their control

- Ensure that no radioactive source or irradiating apparatus is disposed of without the prior approval of the RPS and the RPA, and ensuring that disposal is only undertaken in accordance with agreed and approved procedures

- Arrange for the disposal of all departmental radioactive waste in accordance with approved procedures, and liaising with the RPS as necessary

- Keep adequate records of all radioactive waste disposed of and to submit details to the RPS in accordance with procedures laid down in the department

- Ensure that Risk Assessment and Best Available Techniques documents are completed and approved by the RPS prior to commencement of any new projects involving the use of radiation or radioactive materials within the department

- Ensure that they are aware of the University’s EPR Permit conditions, the General (LR1 and LR2) and Departmental Local Radiation Safety Rules

- Ensure that, whilst working with unsealed sources, they wear suitable and sufficient PPE e.g. gloves, safety glasses, and lab coats etc. and, whilst working with unsealed sources in volatile/powder form, they work only in a fume cupboard

- Take appropriate measures in event of spillages (see Local Rules Part 2 (LR2): Contingency Plans) and advise the RPS accordingly