|  | ### University of Liverpool ### Radiation Protection Office THE USE OF URANIUM AND THORIUM COMPOUNDS |
| --- | --- |

INTRODUCTION

Uranium or thorium compounds are substances or articles in which the U-235 concentration is no more than 0.72% by mass in the case of uranium, and the thorium is in its isotopic proportions found in nature. They include uranium and thorium metals. In the case of uranium, 0.72% is the natural amount of U-235 so this applies to natural and depleted uranium (DU). In the case of thorium the natural isotopic ratios are variable, depending on the origin. The definition applies to chemically processed thorium provided the radionuclide ratio has not been modified for nuclear fuel.

Compounds of natural uranium and thorium have found widespread use in industry and research, notably as pre-treatment agents, in electron microscopy and in in-vitro tracer studies. In the past, safety measures were focussed on the toxicity of uranium, which needed to be assessed in compliance with the Control of Substances Hazardous to Health Regulations 2002 (COSHH) – after all, and regardless of its radioactive properties, uranium is a heavy metal. The radiation hazards associated with these compounds are now addressed by the Ionising Radiations Regulations 2017 (IRR17) and the Environmental Permitting Regulations 2016 (EPR16).

IONISING RADIATION REGULATIONS 2017

The IRR17 require that, without exception, prior risk assessments be carried out for all uses of all sources of ionising radiation. Furthermore, the IRR17 are explicit in requiring that the risk assessments are then used as a basis for doing all that is reasonably practicable to minimise the risks such that radiation exposures are As Low As Reasonably Practicable, this is known as the ALARP principle. To this end, the University of Liverpool requires that all radiation work be controlled such that, under normal operational conditions, no person should receive a radiation exposure in excess of a dose investigation level of 2mSv per year. (By way of comparison, a member of the public receives, on average, an annual dose of 2mSv per year due to natural background radiation, and a person who takes a return air flight to Spain can expect to receive a radiation dose of up to 0.1 mSv). This means that the University believes that, given the nature of teaching and research work undertaken at Liverpool, it is reasonably practicable to restrict all radiation exposures to an intrinsically safe constraint. This has the benefit of protecting all employees, students, visitors, the unborn foetus or breast feeding infants, and will ensure that any person working in accordance with the instructions of local rules and other written procedures will not become exposed to doses of radiation that exceed the variation in natural background radiation levels experienced across Great Britain.

This information sheet presents generic data and derived radiation exposures that may be used in the completion of prior risk assessments.

ENVIRONMENTAL PERMITTING REGULATIONS 2016 (EPR16)

Schedule 23 of the EPR deals with the protection of the environment and general public from the hazards associated with the keeping and use of radioactive materials as well as the discharge or disposal of radioactive waste. EPR requires the University of Liverpool to hold an Open Permit (for unsealed or ‘open’ radioactive materials and waste) plus a Closed Permit (for sealed or ‘closed’ radioactive sources). In their schedules, the Permits specify the limits on how much radioactive material the University can keep and use, plus how much radioactive waste we can accumulate (for how long) or dispose of (via a particular route per time period e.g. per month).

However, for certain small quantities or small concentrations of uranium and thorium compounds, used in a specific way, there are Exemptions to being ‘permitted’ under EPR (as detailed in Parts 6 and 7 of EPR16 Schedule 23). For example an EXEMPTION from holding a ‘Permit’ exists for:

- Up to 5kg (in total) of uranium and thorium compounds as materials and waste

- Up to 500g of solid radioactive waste uranium and thorium compounds disposed of per week to a person who disposes of substantial quantities of non-radioactive waste by burial in landfill, incineration or recovery.

- Up to 500g of aqueous liquid radioactive waste uranium or thorium compounds disposed of per year to a relevant sewer with a normal discharge rate > 100m3 of effluent per day (Note that the main sewer used by the University of Liverpool main campus site routinely handles a daily volume of effluent much greater than this – so this is OK).

- Geological specimens used for exhibition or demonstration purposes only are ‘out of scope’ of the EPR. However, many of them must still be handled in accordance with the IRR17.

The Exemption provisions represent regulatory control but at a lower level of risk than a Permit. The University may use these Exemption provisions so long as the conditions of the Exemption are met, and note that the above conditions and limitations apply to the whole campus not just one particular department.

NOTE: For uranium and thorium compounds, as for all other radioactive sources and waste, accurate records must be maintained by departments of stocks held, and waste disposed of and accumulated at the end of each month. This is to prove to the regulator that we are compliant with the conditions and limitations of either our Permits or the Exemptions.

OCCUPATIONAL EXPOSURE PATHWAYS AND RISK ASSESSMENT

Although the specific activities of unprocessed uranium and thorium are sufficiently low for the substances to be usually controlled by the Exemption provisions, the risks associated with exposure are not small, and potentially very high if the exposure pathway is the inhalation of suspended dusts and aerosols. These radionuclides are so hazardous because they are alpha emitting radionuclides. For example, if inhaled, they would decay in the lungs via a chain of daughters that are themselves alpha emitters: alpha emitting radionuclides deposit large quantities of energy into superficial tissues such as the lining of the alveoli.

The following tables give derived data that quantify the hazards posed by intakes of uranium and thorium compounds through the two exposure pathways of concern.

| Isotope | Specific Activity (Bq/mg) | DPUI a (microSv/Bq) |  |
| --- | --- | --- | --- |
| Natural uranium (99.3% 238U, 0.7% 235U) | 26 | Ingestion | 0.05 |
|  |  | Inhalation | 7.3 |
| Natural thorium (100% 232Th) | 8 | Ingestion | 0.22 |
|  |  | Inhalation | 42 |

a DPUI = Dose Per Unit Intake is the dose to the most radiosensitive organ that would be irradiated via a given exposure pathway i.e. inhalation or ingestion (including skin absorption).

| Isotope | Pathway (route taken into the body) | Derived dose from an intake of 1 mg (microSv) | Intake required to give a dose of 1 mSv (mg) | Dose following the ingestion of 1 ml of a 1% v/v solution b (mSv) |
| --- | --- | --- | --- | --- |
| Natural uranium | Ingestion | 1.2 | 833 | 0.1 |
|  | Inhalation | 190 | 5.3 | NA |
| Natural thorium | Ingestion | 1.8 | 556 | 0.2 |
|  | Inhalation | 336 | 3.0 | NA |

b It is assuming the intake is via ingestion of a drop of the solution spilt onto the skin, which has then been taken in orally. Uranium and thorium are not readily absorbed through the skin on account of their large atomic weight and molecular mass of the compound. For risk assessment purposes assume 10% absorption.

Using the derived exposure data presented above it is possible to determine the level of risk under normal operational conditions.

Arrangements for Work with Uranium and Thorium Compounds

Scope of Arrangements

- These arrangements apply to the use of natural uranium/thorium compounds that are in secular equilibrium (i.e. proportion of daughter products is constant) and which contain:

- 99.3% U-238, 0.7% U-235 and trace amounts of U-234

- Depleted uranium (although the details of the depletion must be obtained prior to purchase)

- 100% Th-230 or Th-232

- Most ‘off-the-shelf’ compounds of uranium or thorium will be in secular equilibrium i.e. uranium nitrate and uranium acetate will be formulated from ‘natural’ uranium.

- Uranium and thorium products that have been physically produced (e.g. by milling) or chemically produced (e.g. by reaction) from naturally occurring minerals may not be covered by this guidance and may require special attention from the RPO. The keeping of geological specimens for exhibition or demonstration purposes is ‘out of scope’ (considered to be not radioactive) with regard to the Environmental Permitting Regulations, however they are, like most other radioactive materials, still subject to the prescriptive requirements of the Ionising Radiations Regulations (IRR) and must be handled with due care and attention in accordance with the IRR.

- Any other isotopes of uranium or thorium may NOT be exempted or ‘out of scope’ with regard to EPR and their potential use must be discussed with the RPO prior to procurement of the material.

Control of Substances

- An Academic Unit may only use uranium and thorium compounds if they have first received permission to do so from the Radiation Protection Office (RPO). The RPO must ensure that the type and amount of radioactive material is compliant the University’s permit conditions, or is exempted from permitting under EPR or ‘out of scope’ i.e. the University is allowed to have it on site. The RPO must also enter uranium and thorium stocks into The University of Liverpool’s EURATOM Safeguards Physical Inventory List.

- The maximum quantity allowed in one Academic Unit is two bottles and a maximum of 100g of compound (unless otherwise approved by the Radiation Protection Office - RPO). Excess quantities, spare or unused stocks can be lodged with the RPO for safekeeping or disposal.

- When not being used, the solid stock and any sub-stocks should be kept in a locked radioactive source store that is suitable labelled. Stocks must not be stored with flammable or corrosive materials that could potentially lead to a loss of containment.

- Aqueous stocks should be held in a secure location, which could be a locked cupboard or a locked lab, and they must be stored such that the risk of spillage is minimised.

- Stained electron microscopy slides must be kept in a safe location i.e. in a suitable container that is itself kept in a locked cupboard or in a cupboard within a locked lab. The store and the container must be marked with a suitable hazard warning sign (see Figure 1).

- A usage log must be maintained that shows:

  - Compound(s) and chemical form

  - Date of purchase or receipt

  - Quantity (mass) of material held

  - Mass of material withdrawn from the stock at any time, and the date of removal.

Working with Uranium and Thorium Compounds

- Work must be carried using a ‘workstation’ area that has been created in a Supervised Radiation Area.

- A ‘workstation’ must be created as follows:

  - A suitably sized area of workbench should be covered with Benchkote (absorbent side up) and taped down using ‘radioactive material’ hazard warning tape. All work, equipment, labware, solutions etc. must be contained within this defined area.

  - The workstation must be provided with drip trays fitted with either disposable liners or Benchkote, and uranium/thorium solutions must only be handled in these trays.

- The workstation must be cleaned (decontaminated) and dismantled immediately after use unless it is in regular use for uranium/thorium work. If the latter is the case, the workstation must be checked for contamination prior to and after use. The Local Rules give further instructions.

- Uranium/thorium compounds may only be handled by persons who received local instruction in the safe uses of these materials. A person who has been approved to do so by the RPS would normally give this training. In particular, the worker must know how to monitor for contamination.

- Uranium and thorium powders must only be handled in a fume cupboard. The only exception to this is where weighing out is to be done using a balance fitted with a measurement chamber. Any spillages must be wiped up using damp tissues. The contaminated tissues must be disposed of as radioactive waste (see below).

- Work must be carried out in strict accordance with the instructions given in the Academic Unit’s Local Rules for the safe use of uranium and thorium compounds. Where not already done, Local Rules for the safe use of uranium and thorium compounds must be drawn up and approved by the RPS and RPA prior to work commencing.

- Great care must be exercised when handling environmental samples containing natural uranium and thorium compounds. The table below gives the maximum quantity of these materials which should be used in teaching experiments. The value quoted for uranium takes account of both chemical toxicity and radiation protection requirements.

| Isotope | Activity (Bq) | Mass (mg) |
| --- | --- | --- |
| Thorium | 5 | 1 |
| Uranium | 50 | 5 |

Waste Disposal

- As with the disposal of other radioactive wastes, the departmental RPS must maintain records of all uranium and thorium waste disposals and via which route.

Aqueous Waste

- All aqueous waste, including spare stock solutions, must be disposed of by decanting into running water in a designated sink. Small volumes of aqueous waste (i.e. less than 50 ml), such as generated by dipping slides into uranium solutions, may be washed down a normal sink, BUT copious amounts of water must be run down the sink immediately after the disposal.

- Alternatively, aqueous wastes can be transferred to the RPO for disposal.

Solid Waste

- All solid waste i.e. small quantities (i.e. less than 1g) of solid uranium or thorium compounds, suppliers empty stock bottles, Benchkote (contaminated or not), gloves, wipes, swabs, tissues, disposable labware, etc. must be sealed into a heavy duty plastic bag which can be disposed of via the normal low level solid radioactive waste route i.e. collected by the RPO. Bags can be kept in a yellow waste bin within the lab.

- Scalpel blades, broken glassware, and unwanted electron microscope slides must also be disposed of via this solid route however such items must first be decontaminated and placed in a sealed cardboard box or sharps box.

- Before disposing, all radiation labels and/or hazard warning tape must be removed and screwed into a ball, or defaced, so that the radiation trefoils etc. can no longer be seen.

- When the bag is full or at 3-monthly intervals (whichever is sooner) the waste must be transferred to the RPO.

- The bag must be labelled as containing uranium and/or thorium solid waste, and an indication of the quantity of solid waste contained in each bag must be supplied to the RPO (e.g. < 5g uranyl acetate).

Recycled Items

- Laboratory glassware used for uranium and/or thorium work may be returned to general circulation only once it has been decontaminated thoroughly down to background levels e.g. by soaking in Decon or equivalent.

The following appendices show two generic risk assessments. You must use your own local arrangements to determine if these are applicable and how to modify them into your local risk assessments and local safety rules.

LOW RADIATION RISK

UNSEALED SOURCES

(URANIUM / THORIUM)

STORED IN THIS CUPBOARD

LOCATION: Sources confined to labelled containers

Figure 1: Typical sign to be affixed to a lockable cupboard used to store uranium or thorium stocks.

APPENDIX 1

RISK ASSESSMENT: USE OF URANIUM AND THORIUM COMPOUNDS IN

IN VITRO STUDIES

Procedure

- Uranium/thorium compounds are made into an aqueous solution. The solid (powder) is weighed out using a balance fitted with a measurement chamber or a balance in a fume cupboard. The solid is transferred into glass in a fume cupboard where it is made up to volume.

- The labelled stock solution is kept on a drip tray in a cupboard, or in some other location, where it is unlikely to get knocked over. Normal precautions are taken to minimise the effect of potential spillages.

- Aliquots are dispensed either in a fume cupboard or on the open bench.

Waste

Before disposing of any uranium and/or thorium wastes, contact the Radiation Protection Office (RPO) with the details of the waste and how you are planning to dispose of it.

All aqueous wastes can usually be disposed of to the designated sink. Note that the Exemption limit for aqueous disposal is approximately 10g per week by this route and this limit applies to the whole University.

Solid wastes must be bagged and labelled. The RPO will collect these for subsequent disposal.

Note: Records of all wastes accumulated and disposed of must be maintained by the departmental RPS.

Major Risks

A. Inhalation of uranium or thorium dust during the weighing out procedures

Route of intake: Inhalation.

Prevention: Weighing out using a balance fitted with a measurement chamber or a balance in a fume cupboard.

B. Contamination of the hands during staining, or following a spillage

Route of intake: Ingestion of contamination transferred from hand to food, drink, cigarettes etc. Note that uranium/thorium will not pass through unbroken skin.

Prevention: Use of nitrile gloves to stop contamination of the hands and washing the hands thoroughly at appropriate times during work and particularly after work.

C. Intake by other persons exposed to contaminated work surfaces, equipment or through the spread of contamination (cross contamination).

Route of intake: Ingestion of contamination inadvertently picked up as a result of cross-contamination then transferred from hand to food, drink, cigarettes etc.

Prevention: Work carried out under (appropriate) containment i.e. in designated areas on surfaces marked out and laid with Benchkote (absorbent side up) and using lined drip trays. Work surfaces and hands are monitored for contamination during and after work. Worker changes gloves frequently or if contamination is suspected.

APPENDIX 2

RISK ASSESSMENT: USE OF URANIUM COMPOUNDS AS ELECTRON MICROSCOPY STAINING AGENTS

Procedure

- Uranium compounds are made into an aqueous solution. The solid (powder) is weighed out using a balance fitted with a measurement chamber or a balance in a fume cupboard. The solid is transferred into glass in a fume cupboard where it is made up to volume (usually to solution strength of between 1 and 3% v/v or occasionally as a saturated solution).

- The labelled stock solution is kept on a drip tray in a cupboard, or in some other location, where it is unlikely to get knocked over. Normal precautions are taken to minimise the effect of potential spillages.

- Slides bearing microscopy specimens are stained by immersing in the uranium solution and air-dried.

Waste

Before disposing of any uranium and/or thorium wastes, contact the Radiation Protection Office (RPO) with the details of the waste and how you are planning to dispose of it.

All aqueous wastes can usually be disposed of to the designated sink. Note that the Exemption limit for aqueous disposal is approximately 10g per week by this route and this limit applies to the whole University.

Solid wastes must be bagged and labelled. The RPO will collect these for subsequent disposal.

Note: Records of all wastes accumulated and disposed of must be maintained by the departmental RPS.

Major Risks

A. Inhalation of uranium or thorium dust during the weighing out procedures

Route of intake: Inhalation.

Prevention: Weighing out using a balance fitted with a measurement chamber or a balance in a fume cupboard.

B. Contamination of the hands during staining, or following a spillage

Route of intake: Ingestion of contamination transferred from hand to food, drink, cigarettes etc. Note that uranium/thorium will not pass through unbroken skin.

Prevention: Use of nitrile gloves to stop contamination of the hands and washing the hands thoroughly at appropriate times during work and particularly after work. Thereafter slides must only be handled when wearing gloves.

C. Intake by other persons exposed to contaminated work surfaces, equipment or through the spread of contamination (cross contamination).

Route of intake: Ingestion of contamination inadvertently picked up as a result of cross-contamination then transferred from hand to food, drink, cigarettes etc.

Prevention: Work carried out under (appropriate) containment i.e. in designated areas on surfaces marked out and laid with Benchkote (absorbent side up) and using lined drip trays. Work surfaces and hands are monitored for contamination during and after work. Worker changes gloves frequently or if contamination is suspected.