|  | # University of Liverpool Radiation Protection Service PRIOR RISK ASSESSMENT FOR THE USE OF NON-IONISING RADIATION (Laser) |
| --- | --- |

| Ref No. of Assessment |  |
| --- | --- |
| Department |  |
| Location & Room No of Laboratory |  |
| Name & Status of Assessor (see Note 1) |  |
| Name of Departmental Safety Officer |  |
| Make and Model of Equipment |  |
| Serial Number of Equipment |  |
| Wavelength of laser beam emitted |  |
| Continuous or pulsed beam |  |
| Laser output power or pulse energy |  |
| Pulse repetition frequency |  |
| Beam diameter and divergence |  |

| Identify the Hazard(s) | (delete as applicable) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Laser | Class 1 | Class 1M | Class 2 | Class 2M | Class 3R | Class 3B | Class 4 |

Describe the process or work procedure

Who might be affected by this work? (see Note 2)

| What is the nature of the risk? |  | Short term/long term? | Mild/severe? |
| --- | --- | --- | --- |
| ## Eye | Photochemical cataract |  |  |
|  | Thermal retinal burn |  |  |
|  | Aqueous flare |  |  |
|  | Corneal burn |  |  |
|  | Other (please specify) |  |  |
| Skin | Burn |  |  |
|  | Other (please specify) |  |  |

| Measures to minimise the risk | Give brief description of existing control measures and indicate if sufficient |
| --- | --- |
| ## Interlocking |  |
| Warning Signs: |  |
| Controlled Area |  |
| Training: |  |
| ## Laser Safety Glasses: |  |

| Laser area requirements: Note: All class 3B and 4 lasers must be used in controlled laser areas. | (Yes – No - N/A) |
| --- | --- |
| Barriers to separate a Laser Controlled Area, such that beams, specular reflections and hazardous diffuse reflections cannot escape into non-controlled areas. |  |
| Laser Controlled Areas should be painted with bright matt colours, this coupled with good lighting ensures eye pupil size is minimised. |  |
| Door entry to a Laser Controlled Area should be interlocked to the laser systems. The interlock should either shut off the laser system when the door is opened or operate an isolation system to contain the beam. An audible warning should be incorporated into any interlock system. For lasers with highly divergent beams it may be sufficient to have an audible warning of entry only, however these should be assessed on an individual basis. |  |
| If necessary, appropriate and suitable laser safety eyewear should be available for each Laser Controlled Area, with a requirement to wear them. |  |
| Each class 3B and 4 laser system should have its own master key switch, which should be removed when the system is not in use. Is there a key-holder policy? |  |
| Warning signs should be placed at the entrance to any Laser Controlled Area |  |
| Any walls/barriers should not be adorned with reflecting surfaces e.g. mirrors. |  |
| Optical tables and work surfaces should be positioned below or above eye level for normal operations (taking into account whether the operator normally sits or stands) |  |
| The laser system and all optics should be securely mounted to the table or work bench |  |
| Beam dumps should be used to terminate beam paths at the end of their useful range. |  |
| Local rules detailing the safe operation of the laser and incident contingency plans should be on prominent display. |  |

Please note any improvements required:

| Additional hazards | (Comment) |
| --- | --- |
| Electrical Hazard: [Danger from power supply, etc.] |  |
| Water Hazard: [Lack of cooling if applicable, effect of burst pipe, etc.] |  |
| Chemical Hazard [Give list] |  |
| Gaseous Hazard: [Asphyxiation, carcinogens produced by the application etc.] |  |
| Fire Hazard [Flammable materials, etc.] |  |
| Other hazard: [specify] |  |

Assessor's Comments:

##### Include any actions taken with dates:

| Assessor's Signature | Printed Name | Date |
| --- | --- | --- |

| Reviewer's Signature | Printed Name | Date |
| --- | --- | --- |

Risk assessment is a five step process:

### 1	Identify the potential hazards

2	Decide who might be harmed and how

3	Evaluate the risks and decide whether existing precautions are adequate

4	Record your findings (i.e. on this form)

- Review the assessment periodically and revise if necessary.

Note 1	The Risk Assessor should normally be the laboratory supervisor but some departments prefer the Departmental Safety Officer to do carry out this task for consistency in risk assessments.

Note 2	Personnel who might be at risk:

1	Registered laser workers

2	Other non-laser workers (including Postgraduates) who share the same laboratory

3	Cleaners

4	Maintenance staff

5	Contractors

6	Visitors

7	Undergraduate students

8	Members of the general public

#### 9	Others

Notes:

Issued 12/10/18	RA2_2018